### 19.1.14.8  Elevator Policy & Procedure

PURPOSE:  To provide facility staff a course of action to follow in the event the elevator should become stuck between floors.

PROCEDURE:

- Obtain the key to open the elevator maintenance room.

- Locate and shut off power to the elevator.  This will return elevator to the ground floor.

- Take key with a red tag, located to the left of the power shut off.

- Put key in hole at the top of the elevator door and turn. This opens the first door.

- Push the latch on the second door and push open at the same time, the person on the elevator can also help push door open.

- Turn on power to the elevator.

- If the power is not restored, push the reset button, which is in the panel on the left.

- If this does not work contact the Maintenance Supervisor, if not available contact the Elevator company at 				.