### 19.1.14.5  Chemical Spills

Purpose:  To inform staff of action to be taken in the event of an outdoor chemical spill.

Policy:  The following action will be taken in the event of an outdoor chemical spill.

- Shut down outside intake ventilation.

- Close all doors to the outside and close and lock all windows.

- Maintenance staff should set all ventilation systems to 100% recirculation so that no outside air is drawn into the building. When this is not possible, ventilation systems should be turned off. This is accomplished by pulling the fire alarm.

- Turn off all heating systems.

- Turn off all air conditioners and switch inlets to the "closed" position. Seal any gaps around window type air conditioners with tape and plastic sheeting, wax paper or aluminum wrap.

- Turn off all exhaust fans in kitchens and bathrooms.

- Close as many internal doors as possible in the building.

- Use tape and plastic food wrapping, wax paper or aluminum wrap to cover and seal bathroom exhaust fan grills, range vents, dryer vents, and other openings to the outside.

- If the gas or vapor is soluble or partially soluble in water, hold a wet cloth over your nose and mouth if gases start to bother you. For a higher degree of protection, go into the bathroom, close the door and turn on the shower in a strong spray to wash the air.

- If an explosion is possible outdoors, close drapes, curtains or shades over windows. Stay away form external windows to prevent injury from flying glass.

- Tune into the Emergency Broadcasting System on the radio or television for further information and guidance.

Law enforcement agencies will make a determination regarding possible evacuation of residents